## HEALTH AND SAFETY INFORMATION PACK

Site Address:

Porthmellon Incinerator Porthmellon Waste Management Site Porthmellon St Mary’s Isles of Scilly TR21 0JY

Project details:

Decommissioning and Demolition of existing Incinerator

Client:

Council of the Isles of Scilly Town Hall St Mary’s Isles of Scilly TR21 0LA

CDM Co-ordinator:

JNC Safety Services Ltd, Woodbine Farm Business Centre, Truro Business Park, Truro, Cornwall TR3 6BW Tel: 01872 262261

INFORMATION PACK DISTRIBUTION

| Date | Issue No | Details | Arch | Eng | PC | Client | Other | Other |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10.03.15 | 1 | Initial Information Pack |  |  | ✓ |  |  |  |

### HEALTH AND SAFETY INFORMATION PACK

### 1.	INTRODUCTION

This document has been prepared in accordance with The Construction (Design and Management) Regulations 2007

The regulations require that a Health & Safety Information Pack be produced which describes the project, existing environment, construction work and timescale, and identifies hazards (including services) and the Safety, Health and Welfare standards required in the execution of the works.

This plan will be updated to incorporate information and any further hazards identified by the designers during the period of the works.

The Principal Contractor must develop the Health & Safety Information Pack into the Construction Phase Plan, incorporating his Programme, Method Statements and detailed arrangements demonstrating his competence to manage the contract safely.

The regulations state that the Client must not permit any contractor to commence any construction operations until the Information Pack and Construction Phase Plan is in place and that welfare arrangements are provided to regulation standard.

A mobilisation period must also be agreed between the Client and the Principal Contractor.

The client will notify the Contractor in writing when the Construction Phase Plan has been accepted and that the Construction work can commence.

The Construction Phase Plan must take account of the following:

- All aspects of Health and Safety relevant to the contractors working on the contract.

- Detailed Health and Safety procedures of all his contractors

- The Health and Safety of persons not connected with the project

- All current relevant legislation

During the Project, the CDM Co-ordinator will prepare a Health and Safety File for issue to the Client upon Practical Completion, for their retention and future use. This file will contain any relevant Health and Safety information relating to the project, including ‘as built’ drawings, operating and maintenance manuals, etc.

The Designer / Principal Contractor / Works Contractors are obliged to supply information to the CDM Co-ordinator whose duty it is to ensure that the Health and Safety Files are produced.

HEALTH AND SAFETY INFORMATION PACK

2.	NATURE OF THE PROJECT.

CLIENT NAME:			Council of the Isles of Scilly

CLIENT ADDRESS:		Town Hall

St Mary’s

Isles of Scilly TR21 0LA

SITE LOCATION:		Porthmellon Incinerator

Porthmellon Waste Management Site

Porthmellon

St Mary’s

Isles of Scilly TR21 0JY

NATURE OF THE WORK:          Decommissioning and Demolition of existing I

ncinerator

PROGRAMME FOR CONSTRUCTION:

Commencement: 	4th May 2015

Mobilisation period:	 3 weeks

Contract Period: 	 8 weeks

3.	THE EXISTING ENVIRONMENT

LOCATION:

The site is that of the existing incinerator located within the fenced waste management site located at Porthmellon. The site is fenced off from residential properties situated on Moor Well Lane.

The existing incinerator is to be decommissioned and then demolished

The waste site lies in an area that is bordered by Porthmellon Industrial Estate, residential properties and other small commercial outlets

ACCESS:

Access to the site is by turning off Telegraph Road into Moor Well Lane; this is a busy road leading to small commercial outlets, residential properties as well as the waste site itself. It is essential that Moor Well Lane is unobstructed throughout these works so that residents can access their properties and the lane is free for use by the emergency services if required.

EXISTING BUILDINGS:

The existing plant is as described in Appendix 1 of the RPS Tender Document

SITE NEIGHBOURS:

Site neighbours are both commercial premises and residential properties; noise, vibration and dust be reduced to a minimum to prevent them becoming a nuisance to residents.

SERVICES:

There are live services on the waste site and in close proximity to the site. These need to be located from Utility companies drawings and from a visual survey to locate manholes, inspection covers, etc. that may indicate the presence of services.

Any services located must be clearly marked and isolated or protected whilst these construction activities take place

GROUND CONTAMINATION:

The ground will have some contamination from the waste stored on the site; therefore it is essential that operatives wear suitable protective clothing e.g. gloves, overalls, respirators whilst working on the site and the contractor must ensure that suitable welfare facilities are provided – see Appendix 4

PRESERVATION ORDERS:

None known

ENVIRONMENTAL RESTRICTIONS:

All efforts are to be made to re-use or re-cycle materials whenever possible.

The usual care must be taken to prevent ground contamination and the pollution of any nearby streams, drains or sewers. Diesel storage tanks must be bunded and driptrays need to be placed under static plant to prevent leakages into the ground; spill kits need to be readily available to deal with any leaks that occur.

4.		DESIGN

The following hazards or work sequences cannot be avoided and must be addressed by the Principal Contractor – (to be read in conjunction with any Design Risk Assessments):

Existing Services.

Hazard:	Contact with existing underground / overhead services on the site or footpath outside the site

(check Utilities’ Drawings)

Work at Height

Hazard:	Falls of Personnel liable to cause personal injury.

Falls of Materials and Equipment.

Access and egress to workplace

Protection of the Public

Hazard:		Unauthorised access into the working areas.

Tripping Hazards.

Dust and Noise.

Lowering of /Disposal of Materials

Hazard:		Dropping of materials in an uncontrolled manner.

Manual Handling

Manual Handling

Hazard:		Handling heavy materials

Environmental Issues

Hazard:		Pollution of Watercourses / Drainage Systems

The Principal Contractor must provide Information prior to the commencement of his activities indicating their proposals for:

- preventing access by unauthorised personnel throughout the course of the project.

- lowering and hoisting materials (including the competent person for preparing the ‘Lifting Plan’)

- working at height including access and egress and designed scaffolds

- ensuring safe traffic management during the project

- ensuring suitable welfare facilities

- ensuring that hazardous substances are handled safely

- preventing fires throughout the construction processes

- ensuring that there is a competent site manager available e.g SMSTS trained

- ensuring that there is a qualified first aider on site at all times

5.	CONSTRUCTION MATERIALS.

The materials to be used in these construction operations are not considered to be particularly hazardous, but COSHH assessments must be available and operatives advised accordingly.

6.	SITE WIDE ELEMENTS

Site hours are limited to 8.00am to 6.00 pm each day and no work to take place on Sundays.

The site is in a Residential area; therefore consideration must be given to dust and noise control at all times to prevent them becoming a nuisance.

There is limited parking on the site; access roads must be kept free at all times for deliveries and / or emergency vehicles.

7.	SITE RULES

The Principal Contractor must:

  - Report any accident or dangerous occurrence to the CDM Co-ordinator.

  - Ensure plant is immobilised during ‘non-working’ hours.

  - Ensure that all manual handling is kept to a minimum; the maximum permitted load is 25 kg, or 20 kg with repetitive tasks. Mechanical means of handling loads must be used when possible

  - Take all necessary steps to comply with all current legislation

  - Demonstrate effective monitoring of his Health and Safety Standards.

  - All persons must wear safety helmets and safety footwear at all times

  - Temporary Roofs, Support, Sheeted and Freestanding Scaffolds must be designed by a competent person

  - Ensure that a fire plan is prepared and fire risk assessments are available to prevent fire on site; a means of raising the alarm must be available, fire-fighting media is to be provided and suitable assembly points are clearly displayed. All site personnel are made aware of these arrangements during the induction given to each operative when commencing on site.

8.	CONTINUING LIAISON

The Principal Contractor must produce a Construction Phase Plan for approval by the Client / CDM Co-ordinator prior to commencing work on site.

The Construction Phase plan must contain Method Statements for dealing with all Health and Safety Matters as described in this document. The Principal Contractor must keep any drawings, manuals, etc and pass them to the CDM Co-ordinator for inclusion in the Health and Safety File.

Copies of all designs for scaffolding, falsework, etc. must be forwarded to the CDM Co-ordinator.

9.	PARTIES TO THE PROJECT.

CLIENT:

Council of the Isles of Scilly

Town Hall

St Mary’s

Isles of Scilly TR21 0LA

Contact: Helen Pearce	Telephone: 01720 424802

DESIGNER:

RPS

2420 The Quadrant

Azrec West

Almondsbury

Bristol BS32 4AQ

Contact:			Telephone: 01291 621821

CDM CO-ORDINATOR:

JNC Safety Services Ltd,

Woodbine Farm Business Centre

Truro Business Park

Truro

Cornwall TR3 6BW

Contact: Neil Caddy		Telephone: 01872 262261

PRINCIPAL CONTRACTOR:

To be appointed

Contact:			Telephone:

CONTRACTORS:

To be appointed

APPENDIX 1 – SAFETY FILE INFORMATION

We will be requiring the following information (as applicable) in order to prepare the Safety File for the Client. We are happy to accept this information as work proceeds, or it may be kept until the project is complete – but it is vital that it is received so that the file can be handed to the Client within one month of project completion. Your final valuation payment may be withheld until we can advise that the file information is complete.

Information required:

- Details of any residual hazards – e.g ground contamination; asbestos-containing materials; etc

- Details (and hazard data sheets) for any hazardous materials used in the construction activities

- Health and Safety Information for any equipment provided for cleaning / maintaining the structure e.g. cradles / access towers, etc

- Location of services – including locations of emergency stop taps, etc; specialist electrical / mechanical contractors must provide a detailed drawing showing layouts

- As-built drawings – marked-up drawings prepared on site or drawings prepared by the Architects.

The information can be provided on written or electronic form (drawings must be pdf format if electronic format provided)

APPENDIX 2 – SERVICES’ DRAWINGS

To follow

APPENDIX 3 – DESIGN RISK ASSESSMENTS

To follow

APPENDIX 4 – WELFARE REQUIREMENTS

MINIMUM WELFARE REQUIREMENTS FOR STATIC SITES

The Client will assist the Principal Contractor where possible by ensuring that a suitable location for welfare facilities is available. The following are the minimum requirements:

- Mess hut with seats and tables – seats to have backrests and tables to have an impervious surface

- Kettle or other means for boiling water and a means of heating food

- Suitable supply of drinking water either from the mains or a bottled supply; containers must be marked as drinking water.

- An adequate supply of toilet facilities, ideally with flushing toilets, with separate facilities for ladies. Toilet paper to be available.

- Washing facilities, with a supply of hot and cold running water; these facilities are to be either in the toilet block or adjacent to them. Soap, towels and nail brushes to be readily available. Washbasins are to be large enough for operatives to wash face, hands and forearms

- A storage area must be provided for operatives clothing; a drying room must be available for operatives to dry their clothes.

- All welfare facilities must be kept clean and tidy with waste bins provided and cleared regularly. Welfare facilities MUST be non-smoking.

- Welfare facilities must be easily accessible and must be adequately heated, lighted and ventilated.

- Mess huts must not be used for the storage of tools and/or materials